Flight Energy Performance

Working notes and reference

Last updated: 11 October 2026

# 1. What this project is

Aircraft broadcast their position every few seconds and volunteers around the world record those signals. OpenSky publishes the result as a monthly file with one row per flight, giving the departure airport, the arrival airport and the aircraft type.

That file says nothing about fuel. This project works the fuel out for flights going in and out of French airports, turns it into CO2, and then puts a price on those emissions under the EU carbon rules.

The question it sets out to answer: which routes, airlines and aircraft types account for the most fuel and CO2 on French traffic, and what would that cost under today's carbon rules?

The scope is the 26 French airports listed in config/airports_fr.yml, and one month of traffic: June 2022. The dataset offers any month from January 2019 to December 2022, so adding another is a single command, but the analysis as it stands covers June 2022 only - 87,903 flights over 30 days. Corsican routes are kept as a separate group, because France has restricted some short domestic flights where the train is a realistic alternative, and Corsica is the obvious case where it is not.

# 2. Why it is being built

This is a portfolio project, built around the three things a data analyst job actually asks for: writing real SQL, defining sensible performance indicators, and dealing with data that cannot be trusted at face value. It also covers a live topic, since airlines are now paying for their emissions under the EU Emissions Trading System.

The same approach works for any moving asset that burns fuel and gets tracked. Ships report the same kind of data through AIS, and the shipping industry has its own version of the same carbon rules.

# 3. Terms worth knowing

The project is full of aviation jargon. These are the terms that come up repeatedly.

| Term | What it means |
| --- | --- |
| ADS-B | Automatic Dependent Surveillance-Broadcast. Aircraft broadcast their position, altitude and speed every few seconds. Anyone with a cheap receiver can pick it up, which is why this data is public. |
| icao24 | A 24-bit code burned into the aircraft's transponder, written as six hex characters such as 3946ab. It identifies the airframe. It can be wrong or duplicated, which is why there are checks for it. |
| Callsign | What the crew uses on the radio, such as AFR1234. The first three letters are the airline - AFR is Air France - so the airline is worked out from this. Private aircraft use their registration instead, which is why those flights have no airline. |
| Registration | The tail number painted on the aircraft, such as F-GKXA. F- means France. |
| ICAO code | A four-letter airport code such as LFPG, used by the flight data. The first letters indicate the region: LF is France, EG is the United Kingdom. |
| IATA code | The three-letter airport code passengers see on tickets, such as CDG. Stored for labelling charts, but the data itself uses ICAO. |
| Type code | A short code for the aircraft model, such as A320 or B738. The fuel model is keyed on this, so a flight without one cannot have its fuel calculated at all. |
| Great circle distance | The shortest distance between two points on a sphere - the straight line an aircraft would fly if airways, weather and airspace restrictions did not exist. Real flights are always longer. |
| Haversine formula | The trigonometry used to turn two pairs of coordinates into a great circle distance. It is written out in SQL in the flight loader. |
| Nautical mile | The unit aviation uses for distance. One nautical mile is 1.852 kilometres. Distances are stored in nautical miles and converted where needed. |
| OpenAP | An open source aircraft performance model from TU Delft. Given an aircraft type and how it is flying, it returns a fuel flow. It is what turns a distance into a fuel figure here. |
| EU ETS | The European Union Emissions Trading System. Airlines must buy an allowance for every tonne of CO2 they emit on covered flights, so emissions have a direct cost. Aviation has been included since 2012. |
| SAF | Sustainable Aviation Fuel. EU rules require a minimum share of it in the fuel uplifted at European airports. It costs considerably more than ordinary jet fuel, so the mandate has a price attached. |
| Star schema | A way of organising tables: one fact table in the middle holding the numbers, surrounded by dimension tables holding the descriptions. It avoids repeating the airport name on every flight row, and it is what Power BI expects. |
| Grain | What one row of a table represents. The grain of fact_flight is one flight. Being clear about grain is what stops numbers being accidentally double counted. |
| PostgreSQL | The database the project runs on. A real server rather than a file, which is how databases work at work, and Power BI connects to it directly. |
| COPY | The PostgreSQL command for loading bulk data. Far faster than inserting rows one at a time, and it can read straight from a stream, which is how the 200 MB flight list gets in. |

# 4. Where the data comes from

Four files. All of them are direct downloads that need no account, no registration and no approval.

## 4.1 Airport reference data

| Publisher | OurAirports, maintained by David Megginson |
| --- | --- |
| Licence | Public domain |
| Size | About 13 MB, roughly 80,000 rows worldwide |
| Used for | Airport names, countries and coordinates |
| Account needed | None needed. Direct download. |

Page:

https://ourairports.com/data/

File:

https://davidmegginson.github.io/ourairports-data/airports.csv

Every distance in the project is calculated from the coordinates in this file, which is why they are downloaded rather than typed in by hand. The file lists every airfield in the world including heliports and closed strips, so only entries with a proper four-letter ICAO code and a real airport type are kept.

## 4.2 Flight list

| Publisher | The OpenSky Network, published through Zenodo |
| --- | --- |
| Licence | Creative Commons, non-commercial |
| Size | 150 to 220 MB compressed per month; 8.1 GB for all 49 months |
| Used for | The flights themselves |
| Account needed | None needed. Direct download. |

Page:

https://zenodo.org/records/7923702

File:

flightlist_YYYYMMDD_YYYYMMDD.csv.gz, one file per month

One row per flight. The file is left compressed on disk. Python unzips it a block at a time and streams it into PostgreSQL with COPY, so the uncompressed gigabyte never has to exist as a file. The columns are:

| Column | What it is |
| --- | --- |
| callsign | Radio callsign, e.g. AFR1234. The airline is the first three letters. |
| number | Commercial flight number, e.g. AF1234. Often blank. |
| icao24 | Transponder code identifying the airframe. |
| registration | Tail number, e.g. F-GKXA. |
| typecode | Aircraft model, e.g. A320. Blank for 37% of flights. |
| origin | Departure airport, ICAO code. Inferred, not scheduled. |
| destination | Arrival airport, ICAO code. Inferred, not scheduled. |
| firstseen | First moment a receiver picked the aircraft up, UTC. |
| lastseen | Last moment a receiver picked the aircraft up, UTC. |
| day | The date the flight belongs to. |
| latitude_1, longitude_1 | Where the aircraft was first seen. |
| latitude_2, longitude_2 | Where the aircraft was last seen. |
| altitude_1, altitude_2 | Altitude at those two points. Not used here. |

## 4.3 Aircraft registry

| Publisher | The OpenSky Network |
| --- | --- |
| Licence | Free to use |
| Size | About 61 MB, 579,435 aircraft |
| Used for | Turning a transponder code into an aircraft type |
| Account needed | None needed. Direct download. |

Page:

https://s3.opensky-network.org/data-samples/metadata/

File:

https://s3.opensky-network.org/data-samples/metadata/aircraft-database-complete-2024-01.csv

This is what fixes the biggest hole in the data. The flight list leaves the aircraft type blank on 37 percent of rows, but every row carries a transponder code, and this file maps those codes to aircraft. Loading it recovered 32,389 of the 32,649 missing types, which took the share of usable flights from 49.6 percent to 77.5.

One quirk: the file's header row is completely blank, so the columns have to be declared by position rather than read by name.

## 4.4 Airline names

| Publisher | OpenFlights |
| --- | --- |
| Licence | Open Database License |
| Size | About 400 KB, 5,774 airlines |
| Used for | Turning airline codes into names and countries |
| Account needed | None needed. Direct download. |

Page:

https://github.com/jpatokal/openflights

File:

https://raw.githubusercontent.com/jpatokal/openflights/master/data/airlines.dat

The flight list has no airline column. The airline is worked out from the first three letters of the callsign - AFR1234 means Air France - and this file turns those codes into names and countries. 323 of the 451 operators in the data are named; the rest are mostly small or recently formed carriers.

## 4.5 Downloading by hand

The pipeline downloads all four files itself, so normally there is nothing to do. To fetch them manually instead, save them to these exact paths and the pipeline will use them rather than downloading again. Do not unzip the second one.

curl -L -o data/raw/ourairports/airports.csv "https://davidmegginson.github.io/ourairports-data/airports.csv"

curl -L -o data/raw/opensky/flightlist_20220601_20220630.csv.gz "https://zenodo.org/records/7923702/files/flightlist_20220601_20220630.csv.gz?download=1"

## 4.6 How to cite them

OurAirports, airport database, public domain. Give the download date.

Strohmeier, M. et al., Crowdsourced air traffic data from the OpenSky Network 2019-2022, Earth System Science Data. Give the Zenodo record and the specific monthly file.

# 5. What the data can and cannot tell you

The flights are reconstructed from transponder signals, not taken from airline schedules. That has consequences, and they are worth stating plainly rather than hiding.

The airports are inferred, not scheduled. OpenSky matches the start and end of each track to the nearest airport. Where receiver coverage is thin this fails, so 11% of the June 2022 flights have no departure airport at all.

The aircraft type is often missing. 37% of the June 2022 flights have no type code, and without it there is no fuel figure. This is the single largest gap in the data.

There is no flight path, only the first and last position. So the distance used is the straight line between the two airports. Real flights are longer, which means every fuel figure is an underestimate until a correction factor is applied.

There is no passenger count. Anything expressed per seat is based on how many seats the aircraft type has, not how many were sold. It measures the efficiency of the capacity offered, not of the people carried.

A small number of rows are exact duplicates - 491 out of 3.17 million in June 2022. They are removed when the file is loaded.

Everything is priced at June 2022 market prices, matching the month of the flight data, so the money figures mean what this traffic cost at the time rather than what it would cost today. The carbon figure is the market value of the emissions, not cash paid: free allocation is deliberately ignored, and in 2022 airlines still received a large share of their allowances free of charge.

# 6. How it fits together

The two downloads go into a PostgreSQL database. The transformations are written in SQL, the KPI formulas live in a SQL view, and Power BI connects to the database directly. Python handles the downloading and the fuel physics, which SQL cannot do.

airports.csv  ---+

+--> PostgreSQL --> SQL view --> Power BI

flightlist.gz ---+

Three choices worth remembering the reasons for:

The transformations are SQL, not pandas. The distance calculation, the KPI formulas and the quality checks all sit in .sql files that can be read, reviewed and tracked in version control.

Power BI only displays things. The calculations already happened in the SQL view, so the DAX stays simple. A formula hidden inside the .pbix file could not be compared between versions.

PostgreSQL. A real database server, the same kind used at work and the one named in most job adverts. Power BI connects to it directly, so the report reads live data rather than importing exported files.

# 7. The commands

| Command | What it does |
| --- | --- |
| winget install -e --id PostgreSQL.PostgreSQL.17 | Install PostgreSQL. Only needed once. |
| uv sync --group dev | Install everything. Downloads Python 3.12 itself. |
| uv run flightperf init | Create the database and all its tables. Safe to rerun. |
| uv run flightperf ingest airports | Download the airport list, 13 MB. |
| uv run flightperf ingest aircraft | Download the aircraft registry, 61 MB. Fills in missing types. |
| uv run flightperf ingest airlines | Download airline names, 400 KB. |
| uv run flightperf ingest flights --year 2022 --month 6 | Download one month of flights, about 200 MB. |
| uv run flightperf build quality | Run the 15 data quality checks. |
| uv run flightperf build fuel | Work out fuel and CO2 for each flight. |
| uv run flightperf build validate | Check the fuel model against published figures. |
| uv run flightperf build kpi | Work out the costs. Needs prices first. |
| uv run flightperf status | Show how many rows are in each table. |
| uv run flightperf sources | Show where every file came from. |
| uv run pytest -q | Run the tests. |
| uv run python scripts/update_summary.py | Rebuild this document. |

# 8. The database

Everything in this section is read straight out of the database when this document is generated, so it always matches what is actually there.

## 8.1 What each table is for

| Table | Kind | Rows | What it holds |
| --- | --- | --- | --- |
| dim_aircraft | table | 6,134 | One row per airframe, built from the flight list. |
| dim_aircraft_type | table | 260 | One row per aircraft model. Gets its performance figures from OpenAP. |
| dim_airport | table | 17,109 | One row per airport, from OurAirports. The coordinates drive every distance in the project. |
| dim_date | table | 1,461 | One row per calendar day. Power BI needs a real date table for its date filters to work. |
| dim_operator | table | 451 | One row per airline, worked out from the first three letters of the callsign. |
| dim_quality_rule | table | 15 | The catalogue of data quality checks, held as data so the dashboard can explain each flag it shows. |
| dim_source | table | 4 | One row per downloaded file: URL, date, size, checksum, row count. |
| fact_flight | table | 87,903 | One row per flight. The centre of the schema and the table everything else describes. |
| fact_flight_quality | table | 31,894 | One row per problem found, per flight. Stored long so that new checks do not need new columns. |
| fuel_curve | table | 1,272 | Fuel against distance for each aircraft type. Each flight is read off this rather than calling the performance model 60,000 times. |
| ref_aircraft | table | 483,017 | The aircraft registry, kept permanently so it can fill in missing aircraft types whenever flights are loaded. |
| ref_airline | table | 5,774 | Airline codes to names, from OpenFlights. |
| v_flight_kpi | view | 87,903 | The view Power BI reads. One row per flight with all the KPIs worked out. |

## 8.2 The columns in each table

dim_aircraft

| Column | Type |
| --- | --- |
| icao24 | character varying |
| registration | character varying |
| type_icao | character varying |
| operator_icao | character varying |
| n_flights_seen | integer |

dim_aircraft_type

| Column | Type |
| --- | --- |
| type_icao | character varying |
| manufacturer | character varying |
| model | character varying |
| engine_type | character varying |
| mtow_kg | double precision |
| oew_kg | double precision |
| typical_seats | integer |
| openap_supported | boolean |
| openap_proxy_type | character varying |
| n_flights_seen | integer |

dim_airport

| Column | Type |
| --- | --- |
| airport_icao | character varying |
| airport_iata | character varying |
| airport_name | character varying |
| municipality | character varying |
| iso_country | character varying |
| iso_region | character varying |
| latitude_deg | double precision |
| longitude_deg | double precision |
| elevation_ft | integer |
| airport_type | character varying |
| is_in_scope | boolean |
| scope_tier | character varying |
| is_eea | boolean |

dim_date

| Column | Type |
| --- | --- |
| date_key | integer |
| full_date | date |
| year | smallint |
| quarter | smallint |
| month | smallint |
| month_name | character varying |
| day_of_month | smallint |
| day_of_week | smallint |
| day_name | character varying |
| is_weekend | boolean |
| iso_week | smallint |
| season_iata | character varying |

dim_operator

| Column | Type |
| --- | --- |
| operator_icao | character varying |
| operator_name | character varying |
| country | character varying |
| is_cargo | boolean |
| is_low_cost | boolean |
| n_flights_seen | integer |

dim_quality_rule

| Column | Type |
| --- | --- |
| rule_code | character varying |
| rule_name | character varying |
| rule_family | character varying |
| severity | character varying |
| description | character varying |
| rationale | character varying |

dim_source

| Column | Type |
| --- | --- |
| source_id | character varying |
| source_name | character varying |
| publisher | character varying |
| source_url | character varying |
| licence | character varying |
| file_name | character varying |
| file_bytes | bigint |
| file_sha256 | character varying |
| downloaded_at_utc | timestamp without time zone |
| rows_loaded | bigint |
| notes | character varying |

fact_flight

| Column | Type |
| --- | --- |
| flight_id | character varying |
| icao24 | character varying |
| callsign | character varying |
| flight_number | character varying |
| registration | character varying |
| operator_icao | character varying |
| type_icao | character varying |
| type_source | character varying |
| origin_icao | character varying |
| destination_icao | character varying |
| date_key | integer |
| first_seen_utc | timestamp without time zone |
| last_seen_utc | timestamp without time zone |
| duration_s | integer |
| first_lat | double precision |
| first_lon | double precision |
| last_lat | double precision |
| last_lon | double precision |
| great_circle_nm | double precision |
| fuel_burn_kg | double precision |
| co2_kg | double precision |
| nox_kg | double precision |
| is_domestic_fr | boolean |
| is_intra_eea | boolean |
| ets_scope_factor | double precision |
| quality_score | double precision |
| quality_status | character varying |
| is_analysis_eligible | boolean |
| source_id | character varying |
| ingested_at_utc | timestamp without time zone |
| fuel_burn_adjusted_kg | double precision |
| co2_adjusted_kg | double precision |
| fuel_cost_eur | double precision |
| ets_cost_eur | double precision |

fact_flight_quality

| Column | Type |
| --- | --- |
| flight_id | character varying |
| rule_code | character varying |
| severity | character varying |
| observed_value | double precision |
| threshold_value | double precision |
| detail | character varying |

fuel_curve

| Column | Type |
| --- | --- |
| type_icao | character varying |
| distance_nm | double precision |
| fuel_kg | double precision |
| airborne_s | double precision |

ref_aircraft

| Column | Type |
| --- | --- |
| icao24 | character varying |
| registration | character varying |
| typecode | character varying |
| manufacturer | character varying |
| model | character varying |
| operator_icao | character varying |

ref_airline

| Column | Type |
| --- | --- |
| operator_icao | character varying |
| operator_name | character varying |
| country | character varying |
| is_active | boolean |

v_flight_kpi

| Column | Type |
| --- | --- |
| flight_id | character varying |
| date_key | integer |
| full_date | date |
| year | smallint |
| month | smallint |
| month_name | character varying |
| day_name | character varying |
| is_weekend | boolean |
| callsign | character varying |
| flight_number | character varying |
| icao24 | character varying |
| registration | character varying |
| operator_icao | character varying |
| operator_name | character varying |
| is_cargo | boolean |
| type_icao | character varying |
| manufacturer | character varying |
| model | character varying |
| typical_seats | integer |
| openap_supported | boolean |
| origin_icao | character varying |
| origin_name | character varying |
| origin_city | character varying |
| origin_country | character varying |
| destination_icao | character varying |
| destination_name | character varying |
| destination_city | character varying |
| destination_country | character varying |
| route_key | text |
| great_circle_nm | double precision |
| great_circle_km | double precision |
| duration_minutes | numeric |
| avg_speed_kt | double precision |
| fuel_burn_kg | double precision |
| co2_kg | double precision |
| co2_tonnes | double precision |
| nox_kg | double precision |
| fuel_kg_per_nm | double precision |
| fuel_kg_per_ask | double precision |
| co2_kg_per_ask | double precision |
| fuel_burn_adjusted_kg | double precision |
| co2_adjusted_kg | double precision |
| co2_adjusted_tonnes | double precision |
| fuel_cost_eur | double precision |
| ets_cost_eur | double precision |
| total_cost_eur | double precision |
| is_domestic_fr | boolean |
| is_intra_eea | boolean |
| ets_scope_factor | double precision |
| ets_chargeable_tonnes | double precision |
| quality_score | double precision |
| quality_status | character varying |
| is_analysis_eligible | boolean |
| source_id | character varying |

# 9. The data quality checks

Before any fuel number is calculated it has to be clear which flights can be trusted. A HARD failure means the flight is dropped from the fuel analysis but still counted. A SOFT failure means it is kept and flagged.

Both counts go in the writeup. Reporting that a given share of flights was dropped, with a breakdown of why, is more convincing than presenting a suspiciously clean dataset.

| Family | Check | Level | What it does | Why it matters |
| --- | --- | --- | --- | --- |
| COMPLETENESS | COM_NO_CALLSIGN | SOFT | The callsign column is blank. | The airline is taken from the callsign, so without it the flight cannot be attributed to an operator. |
| COMPLETENESS | COM_NO_DESTINATION | HARD | The destination column is blank. | Same problem - no distance can be calculated, and the flight cannot be grouped into a route. |
| COMPLETENESS | COM_NO_ORIGIN | HARD | The origin column is blank. | Without both airports there is no distance, and without distance there is no fuel figure. |
| COMPLETENESS | COM_NO_TYPE | HARD | The typecode column is blank. | OpenAP needs an aircraft type to give a fuel figure, so there is nothing to calculate with. |
| CONSISTENCY | CON_DESTINATION_FAR | SOFT | The last point the aircraft was seen at is further from the destination airport than the limit allows. | Same as above, at the other end of the flight. |
| CONSISTENCY | CON_DURATION_LONG | HARD | The flight lasts longer than the maximum in params.yml. | Normally means two separate flights got merged into one row. |
| CONSISTENCY | CON_DURATION_SHORT | HARD | The flight lasts less than the minimum in params.yml. | Usually a fragment of a track that the source failed to join up properly. |
| CONSISTENCY | CON_ORIGIN_FAR | SOFT | The first point the aircraft was seen at is further from the origin airport than the limit allows. | Receiver coverage near the airport was poor, so the departure airport is a guess rather than an observation. |
| CONSISTENCY | CON_SAME_AIRPORT | SOFT | The origin and destination codes are identical. | Real enough - training circuits, test flights, diversions back to base - but not a normal service, so it would distort route averages. |
| CONSISTENCY | CON_SPEED_IMPLAUSIBLE | HARD | Distance divided by duration gives an average speed outside the limits in params.yml. | Either the distance or the duration is wrong, and there is no way to tell which. |
| CONSISTENCY | CON_ZERO_DISTANCE | HARD | The calculated great-circle distance is zero or negative. | Fuel per kilometre cannot be worked out, and it would divide by zero in the KPI view. |
| IDENTITY | ID_AIRPORT_UNKNOWN | HARD | The origin or destination code does not appear in the OurAirports dataset. | No coordinates means no distance. Usually a disused code or a typo in the source. |
| IDENTITY | ID_CALLSIGN_CONFLICT | SOFT | The same callsign is used by two aircraft at overlapping times. | Airlines reuse callsigns through the day, which is fine, but not at the same moment. |
| IDENTITY | ID_DUPLICATE_ICAO24 | HARD | The same icao24 appears on two flights whose times overlap. | One aircraft cannot fly two flights at once, so at least one of the rows is wrong. |
| IDENTITY | ID_TYPE_UNSUPPORTED | SOFT | The type is known but OpenAP does not model it and no similar type was assigned. | Not really an error - but the share of traffic affected still needs reporting. |

# 10. The KPIs

These are the columns the Power BI report is built on. They are all defined in sql/03_marts/001_v_flight_kpi.sql.

| Column | What it is | How it is worked out |
| --- | --- | --- |
| great_circle_nm | Straight-line distance between the two airports | Haversine formula on the airport coordinates |
| fuel_burn_kg | Fuel used, airborne only | OpenAP, from the aircraft type and the distance. No taxi. |
| co2_kg | Carbon dioxide produced | Fuel in kg multiplied by 3.16 |
| fuel_kg_per_nm | Fuel used per nautical mile | fuel_burn_kg divided by great_circle_nm |
| fuel_kg_per_ask | Fuel per available seat-kilometre | Fuel divided by (seats x distance in km). Capacity, not passengers. |
| co2_kg_per_ask | CO2 per available seat-kilometre | Same idea using CO2. The standard way airlines compare efficiency. |
| avg_speed_kt | Average speed over the whole flight | Distance divided by time. Used by one of the quality checks. |
| ets_chargeable_tonnes | CO2 the airline has to buy allowances for | CO2 in tonnes multiplied by the scope factor for that flight |
| quality_score | How far the flight can be trusted, from 0 to 1 | 1.0 if it passed everything; 0 if any hard check failed |
| type_source | Where the aircraft type came from | 'reported' if the flight data had it, 'registry' if looked up |

# 11. What the data looks like so far

How complete the loaded flights are:

| What is missing | Flights | Share |
| --- | --- | --- |
| Aircraft type | 260 | 0.3% |
| Distance (an airport is missing or unknown) | 18,718 | 21.3% |
| Departure airport | 10,052 | 11.4% |
| Arrival airport | 8,358 | 9.5% |
| Airline | 4,151 | 4.7% |
| Total flights loaded | 87,903 | 100% |

The missing aircraft types are the biggest problem, because without a type there is no fuel figure at all. Whether those flights are mostly small private aircraft or full airliners decides how much the totals can be trusted, and that still needs checking.

## 11.1 Busiest routes

| Route | From | To | Flights | Miles |
| --- | --- | --- | --- | --- |
| LFBO-LFPO | Paris (Orly, Val-de-Marne) | Toulouse/Blagnac | 737 | 309.0 |
| LFBO-LFPO | Toulouse/Blagnac | Paris (Orly, Val-de-Marne) | 731 | 309.0 |
| LFMN-LFPO | Nice, Alpes-Maritimes | Paris (Orly, Val-de-Marne) | 646 | 365.0 |
| LFMN-LFPO | Paris (Orly, Val-de-Marne) | Nice, Alpes-Maritimes | 575 | 365.0 |
| EGLL-LFPG | London | Paris (Roissy-en-France, Val-d'Oise) | 375 | 188.0 |
| EGLL-LFPG | Paris (Roissy-en-France, Val-d'Oise) | London | 361 | 188.0 |
| LFPO-LPPT | Lisbon | Paris (Orly, Val-de-Marne) | 354 | 776.0 |
| LEMD-LFPO | Paris (Orly, Val-de-Marne) | Madrid | 348 | 555.0 |
| LEMD-LFPO | Madrid | Paris (Orly, Val-de-Marne) | 348 | 555.0 |
| LEBL-LFPG | Barcelona | Paris (Roissy-en-France, Val-d'Oise) | 346 | 463.0 |

## 11.2 Busiest airlines and aircraft types

| Airline | Flights | Aircraft type | Flights |
| --- | --- | --- | --- |
| AFR | 23,479 | A320 | 19,104 |
| EJU | 6,758 | B738 | 15,193 |
| TVF | 6,075 | A319 | 9,799 |
| RYR | 5,791 | A321 | 5,998 |
| VLG | 3,220 | E190 | 3,609 |
| VOE | 2,487 | B77W | 2,467 |
| DLH | 1,855 | A20N | 2,210 |
| EZY | 1,533 | A318 | 1,821 |
| BAW | 1,181 | A21N | 1,590 |
| FDX | 1,163 | BCS3 | 1,527 |

# 12. Which files have been used

Recorded automatically every time something is downloaded, so it is possible to prove later exactly which version of a dataset produced a given number. The same information is shown by 'uv run flightperf sources'.

| Name | Publisher | File | Size | Rows | Downloaded | Checksum |
| --- | --- | --- | --- | --- | --- | --- |
| flightlist_202206 | The OpenSky Network (via Zenodo) | flightlist_20220601_20220630.csv.gz | 219.3 MB | 87,903 | 2026-09-23 | 1c3d3caadf88... |
| openflights_airlines | OpenFlights | airlines.dat | 0.4 MB | 5,774 | 2026-10-11 | 39be1a432e8b... |
| opensky_aircraft | The OpenSky Network | aircraft-database.csv | 61.5 MB | 483,017 | 2026-10-11 | db72e0cb6567... |
| ourairports | OurAirports (David Megginson) | airports.csv | 12.7 MB | 17,109 | 2026-09-23 | e52f4e19b23c... |

# 13. The assumptions

Every number that can be tweaked lives in config/params.yml rather than in the code, so the assumptions can be checked in one place and tested for how much they change the answer.

Assumptions that have not been verified are deliberately left empty, and the code refuses to run the steps that need them. A missing carbon price treated as zero would produce a confident, wrong answer with no warning at all, which is the worst thing this project could do.

# 14. Where each part has got to

| Part | Status | Notes |
| --- | --- | --- |
| Database and tables | Done | PostgreSQL, star schema |
| Airport data | Done | 17,109 airports from OurAirports |
| Flight data | Done | 87,903 French flights for June 2022 |
| Aircraft registry | Done | Recovers 32,389 missing aircraft types |
| Airline names | Done | 323 of 451 operators named |
| Source tracking | Done | URL, date, size and checksum recorded per file |
| Data quality checks | Done | 15 checks; 77.5 percent of flights usable |
| Fuel model | Done | OpenAP; fuel and CO2 for 59,772 flights |
| Model validation | Done | 7 of 8 types within 15 percent of published |
| Carbon costs | Done | Priced at June 2022 market prices, sources cited |
| Power BI report | Done | Five pages, built on core.v_flight_kpi |

# 15. Progress log

| Date | Step | What was done |
| --- | --- | --- |
| 2026-09-17 | Project setup | Decided what the project is, picked the tools, created the repository. |
| 2026-09-17 | Database and schema | Built the DuckDB database with a star schema, the date table, and the list of data quality checks. |
| 2026-09-18 | Cut the scope down | The first plan needed seven data sources, three of them behind a registration or an access application. Replaced the lot with two plain CSV downloads that need no account. Dropped the trajectory tables, because the new source is already one row per flight. |
| 2026-09-18 | Airport data loaded | Loaded 17,109 airports from OurAirports, including all 26 French airports in scope. Added a dim_source table recording the URL, download date, size and checksum of every file used. |
| 2026-09-18 | Flight data loaded | Loaded June 2022. 3,174,252 flights worldwide in the file, of which 87,903 touch a French airport. |
| 2026-09-18 | Moved to PostgreSQL | Switched from DuckDB to PostgreSQL: it is what job adverts ask for, and Power BI connects to it directly, which removed the Parquet export step entirely. The 200 MB flight list is streamed in with COPY, unzipped a block at a time. Deleted the leftover data/curated and data/interim folders. |
| 2026-09-18 | PostgreSQL verified end to end | Installed PostgreSQL 17, rebuilt the database from scratch and reloaded June 2022. Every figure matches what DuckDB produced, so nothing broke in the move. The load takes 52 seconds and the database is 35 MB, down from 640 MB, because staging tables are now dropped once the load finishes. |
| 2026-09-23 | Quality checks running | Turned the 15 checks into code. Every flight now gets a verdict and a score, and every problem found is recorded as its own row, so the dashboard can break it down by reason. Nothing is deleted. On the first run only 49.6 percent of flights were usable, the biggest cause being 37 percent with no aircraft type. |
| 2026-09-23 | Aircraft registry added | Every flight carries a transponder code even when the aircraft type is blank, so a 61 MB registry from OpenSky was loaded to look the missing types up. It recovered 32,389 of the 32,649, and usable flights went from 49.6 to 77.5 percent. |
| 2026-09-23 | Airline names added | A 400 KB file from OpenFlights turns operator codes into names, so the dashboard shows Air France rather than AFR. 323 of the 451 operators in the data are named. |
| 2026-09-23 | Ingest made order-independent | Loading the aircraft registry before the flights used to do nothing, because there was nothing yet to fix. The reference data is now kept in its own tables, so whichever order the commands run in, the result is the same. |
| 2026-09-23 | Fuel model built | OpenAP now gives fuel and CO2 for every flight that passed the checks. Rather than calling the model 60,000 times, a fuel-against-distance curve is built per aircraft type and each flight is read off it. June 2022 comes to 543,054 tonnes of fuel across 59,772 flights, before the route correction. |
| 2026-09-23 | Model checked against published figures | Modelled cruise burn was compared with published figures for eight aircraft types. Seven agree within 15 percent. The Boeing 777-300ER comes out 31 percent high, which is reported rather than hidden - it matters because long-haul flights dominate the CO2 total. |
| 2026-09-26 | Prices researched and applied | The carbon price, jet fuel price and route-extension factor were looked up and written into params.yml, each with its source and the date it was read. All three are June 2022 figures, matching the flight data. June 2022 works out at 559,128 tonnes of fuel and 1.77 million tonnes of CO2, worth EUR 730 million in fuel and EUR 38 million in carbon. The striking result: 74 percent of the CO2 falls outside the EU carbon market, because the long-haul flights that dominate emissions are not covered. |
| 2026-09-23 | Cost step written, waiting on prices | The code that turns CO2 into money is written and wired up. It refuses to run until the carbon price, the fuel price and the route-extension factor are filled into params.yml, and prints exactly what is missing. |
| 2026-09-18 | Three bugs found and fixed | The ~ operator is not a regular expression match in DuckDB, so the airline column was coming out almost entirely empty; regexp_matches fixed it. least() and greatest() skip NULLs, so a flight missing an airport produced a nonsense route like LFPG-LFPG. And the source file contains 491 exactly duplicated rows, now removed on load. |
| 2026-10-11 | Power BI dashboard built | Five pages on top of core.v_flight_kpi: overview, routes and airports, airlines and aircraft, carbon cost, and data quality. Every KPI counts only flights that passed the quality checks. Power BI imports straight from PostgreSQL, so a pipeline rerun plus Refresh brings it up to date. |
| 2026-10-11 | Three bugs the dashboard exposed | Aircraft types were named after the first registry entry alphabetically, so a misfiled glider renamed every B737; each type now takes the most common name. No airline was flagged as cargo, because loading flights wiped the flag and the name check missed 'Federal Express'; cargo is now also spotted by ICAO code (11 operators, 3,229 flights). And reloading aircraft data after the fuel step crashed on a foreign key; the rebuild now updates in place. Fuel, CO2 and cost totals were unchanged. |

# 16. What comes next

Write up the finding: 74 percent of the CO2 from French air traffic falls outside the EU carbon market, because the long-haul flights that dominate emissions are not covered.

Settle whether flights to the UK and Switzerland were covered by the EU carbon market in 2022. It moves the 74 percent figure directly.

Cite the eight cruise-burn figures in config/validation_reference.yml.

Load June 2019 as well, to compare a pre-COVID month against June 2022.

# 17. Things still to sort out

Whether flights from the EEA to the UK and Switzerland were covered by the EU carbon market in 2022. Switzerland is currently counted as inside, the UK as outside, and neither has a source. It moves the headline finding that 74 percent of the CO2 is uncovered.

The route correction uses EUROCONTROL's en-route figure, which excludes the terminal area around each airport. The full gate-to-gate extension is larger, so fuel is still understated.

The Boeing 777-300ER is modelled 31 percent above its published cruise burn. Long-haul flights dominate the CO2 total, so this pushes the headline figure up. Worth either finding a better reference or stating the effect on the total.

Taxi fuel is not counted at all, because OpenAP has no model for engines at idle. Real block fuel is a few percent higher than these figures.

18,410 flights are still dropped for having no departure or arrival airport. Unlike the missing aircraft types, there is no reference file that can fill these in.

The published cruise figures in config/validation_reference.yml are working numbers and still need proper citations.

Switzerland is counted as part of the European Economic Area in the code, because it runs a linked carbon market. Strictly it is not in the EEA. That choice needs stating in the writeup.